

Housing Devices, Inc.

ADA 121® Audio Visual 
Alert System
(For the Hearing & Visually Impaired)
STANDARD FEATURES:

(
Vandal Proof 18ga. 304 Stainless Steel 
· 
· 
· 

· 
· Dual Audio & Visual Alarms for the Hearing & Visual Impaired

· Adjustable Timer (Alarm)

· Push Button Rest Switch

· Push button On-Off Switch

· 
· Flashing Strobe Light (110 Candela)
· High Volume (90dbA) Piezoelectric alarm
(
16 Vac Wall Mounted Plug-In Transformer or 16Vac Hardwired Transformer
· Surface Mount
· Flush Mount






· 


· 
· 




(
Remote Units available for Multiple Location installations 
(
Telephone Interface



(  Green power LED
· Dual LED (door or phone) 
DESCRIPTION:

The ADA 121® is a modular device which enables people with visual and/or hearing impairments to be made aware of incoming telephone calls and/or when visitors to their residence are outside their front door and ringing the doorbell.

Resident awareness is stimulated via a very loud piezoelectric buzzer and a  flashing 

strobe light.  The length of the sound emitted and the strobe flashing may be adjusted from 1-60 seconds and can be RESET anytime in between.

The ADA 121® is designed to work with existing residential or building access control systems including: doorbell, intercom, telephone entry, and emergency call systems; as well as the standard telephone system (POTS).




[image: image1.wmf]Reset

On/Off

Door

Phone

Power

ADA-121
4.75”h x 6.5”w x .375”d
SYSTEM OPERATION: 

Pressing the ON/OFF button once will turn on the unit and power LED and then pressing it again will turn off the unit.  When activated the unit will sound a 90dbA piezoelectric buzzer and illuminate a 110 candela flashing strobe light.  Pressing the RESET button will stop the buzzer and strobe and reset the system for the next alarm. 
Activation is accomplished by pushing the residence’s doorbell, or depressing the appropriate residence intercom button, a telephone entry system, or by dialing the residence telephone number from a standard telephone system.









Each time the residence is contacted the ADA 121’s buzzer will sound and the flashing strobe light will illuminate, notifying the unit’s resident that someone is trying to contact them.  
Depressing the “RESET” button on the unit resets the ADA 121.  

Up to 5 ADA 121® remote units may be used together in a single system.
































Mounting SYSTEMS & Dimensions




















The ADA-121 is designed to be mounted
surface or flush using a 3 gang old or new 
work box (flush mount), or a 3 gang Wiremold 
box (surface mount)






 
RESET











POWER





  TALK      LISTEN       TTROUBLE





     





                    





                





1ST FLOOR 





     





NORTH 





1ST FLOOR





 





SOUTH 





    





2ND FLOOR     





NORTH 





     








     








2ND FLOOR


SOUTH





     





ELEV LOBBY





3





3





3





2





4





1





Master Station





Relay Card Cabinet





with





Control Board





Relay Cards





Battery Back Up Backup Cabinet


With


ADA-35 Amplifier


ADA-24V transformer


Battery Charger


Batteries

















OR


Amplifier Cabinet


With


ADA – 35 Amplifier





110VAC Power





PUSH 





FOR





HELP





HELP   COMING





HELP   REQUESTED





PUSH 





FOR





HELP





HELP   COMING





HELP   REQUESTED





PUSH 





FOR





HELP





HELP   COMING





HELP   REQUESTED





Area Station





Area Station





Area Station





5








© Housing Devices, Inc.
      HDI©
           800-392-5200
www.housingdevices.com
© Housing Devices, Inc.
                                     HDI©
           800-392-5200

www.housingdevices.com
© 1998 Housing Devices, Inc.

Page 3
800-392-5200

_1220422663.vsd
Reset�

On/Off�

Door�

Phone�

Power�


_1267427364.vsd
�

�

Phone 1
INCOMING LINE�

Phone 2
TO PHONE�

TIME �ADJUST
1-60 sec.�

DOOR
TONE�

1�

2�

3�

4�

5�

6�

7�

8�

9�

10�

PHONE
TONE�

ADJUST SWITCHS 1-3
ON OR OFF 
FOR DIFFERENT
ALERT TONES�

ADJUST SWITCHS 1-3
ON OR OFF 
FOR DIFFERENT
ALERT TONES�

HOUSING DEVICES, INC.
ADA-121 AUDIO VISUAL ALERT SYSTEM INSTALLATION INSTRUCTIONS �

Housing Devices, Inc    407 Rear Mystic Ave.  Medford, MA 02155 Tel (800) 392-5200  Fax (781) 395-8000
www.housingdevices.com �

Terminal Block 1-10 Connections�

Terminal 1  16 Vac input �

Timer Adjustment�

Terminal 10 Remote Strobe Connection (-) Neg.�

Terminal 9  Remote Strobe Connection (+) Pos.  �

Terminal 8  24 Vdc input (-) Neg �

Terminal 7  Apartment Station Connection (Blue wire) �

Terminal 6  Apartment Station Connection (Black wire)  �

Terminal 5  Doorbell switch connection �

Terminal 4  Doorbell switch connection �

Terminal 3  (Not used) �

Terminal 2  16 Vac input or 24 Vdc (+) Pos.�

Phone & Door
Tone Adjustment�

Telephone Connection�

�

The timer adjustment is set by using the round white pot mounted on the top right corner of the circuit board.�

�

Turn
Left�

Turn
Right�

Decrease Time�

Increase Time�

Connect the incoming phone line to  the (Phone 1) INCOMING LINE RJ 11 Modular Jack, Then connect the line from the telephone to the (phone 2) TO PHONE RJ 11 Modular Jack �

The ADA-121 uses 2 DIP switches to adjust the sound of the tone for the DOOR & PHONE six different audio tones can be heard  by turning either of the three switches on or off (the volume of the tone is not adjustable)�


